## Types of instructions

## Depending on operation they perform, all instructions are divided in several groups:

- Arithmetic Instructions

- Branch Instructions

- Data Transfer Instructions

- Data Manipulation Instructions

- Logic Instructions

- Bit-oriented Instructions

- The first part of each instruction, called MNEMONIC refers to the operation an instruction performs (copy, addition, logic operation etc.).

- Mnemonics are abbreviations of the name of operation being executed. For example:

- INC R1 - Means: Increment register R1 (increment register R1);

- LJMP LAB5 - Means: Long Jump LAB5 (long jump to the address marked as LAB5);

- JNZ LOOP - Means: Jump if Not Zero LOOP (if the number in the accumulator is not 0, jump to the address marked as LOOP);

- The other part of instruction, called OPERAND is separated from mnemonic by at least one whitespace and defines data being processed by instructions.

- Some of the instructions have no operand, while some of them have one, two or three.

- If there is more than one operand in an instruction, they are separated by a comma. For example:

- RET - return from a subroutine;

- JZ TEMP - if the number in the accumulator is not 0, jump to the address marked as TEMP;

- ADD A,R3 - add R3 and accumulator;

- CJNE A,#20,LOOP - compare accumulator with 20. If they are not equal, jump to the address marked as LOOP;

Arithmetic instructions:

Arithmetic instructions perform several basic operations such as addition, subtraction, division, multiplication etc.

After execution, the result is stored in the first operand. For example:

ADD A,R1 - The result of addition (A+R1) will be stored in the accumulator.

| ARITHMETIC INSTRUCTIONS |  |  |  |
| --- | --- | --- | --- |
| Mnemonic | Description | Byte | Cycle |
| ADD A,Rn | Adds the register to the accumulator | 1 | 1 |
| ADD A,direct | Adds the direct byte to the accumulator | 2 | 2 |
| ADD A,@Ri | Adds the indirect RAM to the accumulator | 1 | 2 |
| ADD A,#data | Adds the immediate data to the accumulator | 2 | 2 |
| ADDC A,Rn | Adds the register to the accumulator with a carry flag | 1 | 1 |
| ADDC A,direct | Adds the direct byte to the accumulator with a carry flag | 2 | 2 |
| ADDC A,@Ri | Adds the indirect RAM to the accumulator with a carry flag | 1 | 2 |
| ADDC A,#data | Adds the immediate data to the accumulator with a carry flag | 2 | 2 |
| SUBB A,Rn | Subtracts the register from the accumulator with a borrow | 1 | 1 |
| SUBB A,direct | Subtracts the direct byte from the accumulator with a borrow | 2 | 2 |
| SUBB A,@Ri | Subtracts the indirect RAM from the accumulator with a borrow | 1 | 2 |
| SUBB A,#data | Subtracts the immediate data from the accumulator with a borrow | 2 | 2 |
| INC A | Increments the accumulator by 1 | 1 | 1 |
| INC Rn | Increments the register by 1 | 1 | 2 |
| INC Rx | Increments the direct byte by 1 | 2 | 3 |
| INC @Ri | Increments the indirect RAM by 1 | 1 | 3 |
| DEC A | Decrements the accumulator by 1 | 1 | 1 |
| DEC Rn | Decrements the register by 1 | 1 | 1 |
| DEC Rx | Decrements the direct byte by 1 | 1 | 2 |
| DEC @Ri | Decrements the indirect RAM by 1 | 2 | 3 |
| INC DPTR | Increments the Data Pointer by 1 | 1 | 3 |
| MUL AB | Multiplies A and B | 1 | 5 |
| DIV AB | Divides A by B | 1 | 5 |
| DA A | Decimal adjustment of the accumulator according to BCD code | 1 | 1 |

#### Branch Instructions:

There are two kinds of branch instructions:

Unconditional jump instructions: upon their execution a jump to a new location from where the program continues execution is executed.

Conditional jump instructions: a jump to a new program location is executed only if a specified condition is met. Otherwise, the program normally proceeds with the next instruction.

| BRANCH INSTRUCTIONS |  |  |  |
| --- | --- | --- | --- |
| Mnemonic | Description | Byte | Cycle |
| ACALL addr11 | Absolute subroutine call | 2 | 6 |
| LCALL addr16 | Long subroutine call | 3 | 6 |
| RET | Returns from subroutine | 1 | 4 |
| RETI | Returns from interrupt subroutine | 1 | 4 |
| AJMP addr11 | Absolute jump | 2 | 3 |
| LJMP addr16 | Long jump | 3 | 4 |
| SJMP rel | Short jump (from –128 to +127 locations relative to the following instruction) | 2 | 3 |
| JC rel | Jump if carry flag is set. Short jump. | 2 | 3 |
| JNC rel | Jump if carry flag is not set. Short jump. | 2 | 3 |
| JB bit,rel | Jump if direct bit is set. Short jump. | 3 | 4 |
| JBC bit,rel | Jump if direct bit is set and clears bit. Short jump. | 3 | 4 |
| JMP @A+DPTR | Jump indirect relative to the DPTR | 1 | 2 |
| JZ rel | Jump if the accumulator is zero. Short jump. | 2 | 3 |
| JNZ rel | Jump if the accumulator is not zero. Short jump. | 2 | 3 |
| CJNE A,direct,rel | Compares direct byte to the accumulator and jumps if not equal. Short jump. | 3 | 4 |
| CJNE A,#data,rel | Compares immediate data to the accumulator and jumps if not equal. Short jump. | 3 | 4 |
| CJNE Rn,#data,rel | Compares immediate data to the register and jumps if not equal. Short jump. | 3 | 4 |
| CJNE @Ri,#data,rel | Compares immediate data to indirect register and jumps if not equal. Short jump. | 3 | 4 |
| DJNZ Rn,rel | Decrements register and jumps if not 0. Short jump. | 2 | 3 |
| DJNZ Rx,rel | Decrements direct byte and jump if not 0. Short jump. | 3 | 4 |
| NOP | No operation | 1 | 1 |

#### Data Transfer Instructions:

- Data transfer instructions move the content of one register to another.

- The register the content of which is moved remains unchanged.

- If they have the suffix “X” (MOVX), the data is exchanged with external memory.

| DATA TRANSFER INSTRUCTIONS |  |  |  |
| --- | --- | --- | --- |
| Mnemonic | Description | Byte | Cycle |
| MOV A,Rn | Moves the register to the accumulator | 1 | 1 |
| MOV A,direct | Moves the direct byte to the accumulator | 2 | 2 |
| MOV A,@Ri | Moves the indirect RAM to the accumulator | 1 | 2 |
| MOV A,#data | Moves the immediate data to the accumulator | 2 | 2 |
| MOV Rn,A | Moves the accumulator to the register | 1 | 2 |
| MOV Rn,direct | Moves the direct byte to the register | 2 | 4 |
| MOV Rn,#data | Moves the immediate data to the register | 2 | 2 |
| MOV direct,A | Moves the accumulator to the direct byte | 2 | 3 |
| MOV direct,Rn | Moves the register to the direct byte | 2 | 3 |
| MOV direct,direct | Moves the direct byte to the direct byte | 3 | 4 |
| MOV direct,@Ri | Moves the indirect RAM to the direct byte | 2 | 4 |
| MOV direct,#data | Moves the immediate data to the direct byte | 3 | 3 |
| MOV @Ri,A | Moves the accumulator to the indirect RAM | 1 | 3 |
| MOV @Ri,direct | Moves the direct byte to the indirect RAM | 2 | 5 |
| MOV @Ri,#data | Moves the immediate data to the indirect RAM | 2 | 3 |
| MOV DPTR,#data | Moves a 16-bit data to the data pointer | 3 | 3 |
| MOVC A,@A+DPTR | Moves the code byte relative to the DPTR to the accumulator (address=A+DPTR) | 1 | 3 |
| MOVC A,@A+PC | Moves the code byte relative to the PC to the accumulator (address=A+PC) | 1 | 3 |
| MOVX A,@Ri | Moves the external RAM (8-bit address) to the accumulator | 1 | 3-10 |
| MOVX A,@DPTR | Moves the external RAM (16-bit address) to the accumulator | 1 | 3-10 |
| MOVX @Ri,A | Moves the accumulator to the external RAM (8-bit address) | 1 | 4-11 |
| MOVX @DPTR,A | Moves the accumulator to the external RAM (16-bit address) | 1 | 4-11 |
| PUSH direct | Pushes the direct byte onto the stack | 2 | 4 |
| POP direct | Pops the direct byte from the stack/td> | 2 | 3 |
| XCH A,Rn | Exchanges the register with the accumulator | 1 | 2 |
| XCH A,direct | Exchanges the direct byte with the accumulator | 2 | 3 |
| XCH A,@Ri | Exchanges the indirect RAM with the accumulator | 1 | 3 |
| XCHD A,@Ri | Exchanges the low-order nibble indirect RAM with the accumulator | 1 | 3 |

#### Logic Instructions:

- Logic instructions perform logic operations upon corresponding bits of two registers.

- After execution, the result is stored in the first operand.

| LOGIC INSTRUCTIONS |  |  |  |
| --- | --- | --- | --- |
| Mnemonic | Description | Byte | Cycle |
| ANL A,Rn | AND register to accumulator | 1 | 1 |
| ANL A,direct | AND direct byte to accumulator | 2 | 2 |
| ANL A,@Ri | AND indirect RAM to accumulator | 1 | 2 |
| ANL A,#data | AND immediate data to accumulator | 2 | 2 |
| ANL direct,A | AND accumulator to direct byte | 2 | 3 |
| ANL direct,#data | AND immediae data to direct register | 3 | 4 |
| ORL A,Rn | OR register to accumulator | 1 | 1 |
| ORL A,direct | OR direct byte to accumulator | 2 | 2 |
| ORL A,@Ri | OR indirect RAM to accumulator | 1 | 2 |
| ORL direct,A | OR accumulator to direct byte | 2 | 3 |
| ORL direct,#data | OR immediate data to direct byte | 3 | 4 |
| XRL A,Rn | Exclusive OR register to accumulator | 1 | 1 |
| XRL A,direct | Exclusive OR direct byte to accumulator | 2 | 2 |
| XRL A,@Ri | Exclusive OR indirect RAM to accumulator | 1 | 2 |
| XRL A,#data | Exclusive OR immediate data to accumulator | 2 | 2 |
| XRL direct,A | Exclusive OR accumulator to direct byte | 2 | 3 |
| XORL direct,#data | Exclusive OR immediate data to direct byte | 3 | 4 |
| CLR A | Clears the accumulator | 1 | 1 |
| CPL A | Complements the accumulator (1=0, 0=1) | 1 | 1 |
| SWAP A | Swaps nibbles within the accumulator | 1 | 1 |
| RL A | Rotates bits in the accumulator left | 1 | 1 |
| RLC A | Rotates bits in the accumulator left through carry | 1 | 1 |
| RR A | Rotates bits in the accumulator right | 1 | 1 |
| RRC A | Rotates bits in the accumulator right through carry | 1 | 1 |

#### Bit-oriented Instructions:

- Similar to logic instructions, bit-oriented instructions perform logic operations. The difference is that these are performed upon single bits.

| BIT-ORIENTED INSTRUCTIONS |  |  |  |
| --- | --- | --- | --- |
| Mnemonic | Description | Byte | Cycle |
| CLR C | Clears the carry flag | 1 | 1 |
| CLR bit | Clears the direct bit | 2 | 3 |
| SETB C | Sets the carry flag | 1 | 1 |
| SETB bit | Sets the direct bit | 2 | 3 |
| CPL C | Complements the carry flag | 1 | 1 |
| CPL bit | Complements the direct bit | 2 | 3 |
| ANL C,bit | AND direct bit to the carry flag | 2 | 2 |
| ANL C,/bit | AND complements of direct bit to the carry flag | 2 | 2 |
| ORL C,bit | OR direct bit to the carry flag | 2 | 2 |
| ORL C,/bit | OR complements of direct bit to the carry flag | 2 | 2 |
| MOV C,bit | Moves the direct bit to the carry flag | 2 | 2 |
| MOV bit,C | Moves the carry flag to the direct bit | 2 | 3 |