- Consider a disk with 32 platters each with 2 recording surfaces. There are 128 tracks per surface and 32 sectors per track. Each sector has equal capacity of 1KB

- Calculate

1.Number of surfaces in disk:

2. Number of tracks on disk:

3. Number of sectors in disk:

4. Number of bytes on disk:

5. Number of bits for disk addressing:

- A hard disk has 16 sectors per track, 4 platters each with 2 recording surfaces and 32 cylinders. The address of a sector is given as a triple <c,h,s>, where c is the cylinder number, h is the surface number and s is the sector number. Thus, the 0th sector is addressed as <0,0,0>, the 1st sector as <0,0,1>, and so on.

- The address <12, 6,12> corresponds to sector number?

- A hard disk has 16 sectors per track, 4 platters each with 2 recording surfaces and 32 cylinders. The address of a sector is given as a triple <c,h,s>, where c is the cylinder number, h is the surface number and s is the sector number. Thus, the 0th sector is addressed as <0,0,0>, the 1st sector as <0,0,1>, and so on.

- Find the address of 867th Sector..

- Consider a disk with 12 platters, 2 surfaces per platter, 3K tracks per surface, 6K sectors per track and 2048 Bytes per sector. Disk rotates with 3000rpm. Seek time is 6ms. Find disk access time?

- A disk has each track with 1k sectors each with 4KB capacity and disk has 6000RPM. The transfer rate of the disk is?